# ASIL Classification Standard

ISO 26262-3 Severity (S) · Exposure (E) · Controllability (C) — and worked ACC / HA-LCA / VDD classification

Automotive Safety Integrity Level (ASIL) is derived per ISO 26262-3 from three risk parameters — Severity (S), Exposure (E), and Controllability (C) — assessed for each hazardous event at the vehicle level, then allocated down to the software requirements that implement the associated safety function. Not every feature is safety-related: functions with no credible hazard path (e.g. a display-mode arbitration block) are classified QM (Quality Managed) rather than assigned an ASIL.

# 1. Severity (S)

| Class | Definition |
| --- | --- |
| S0 | No injuries |
| S1 | Light to moderate injuries |
| S2 | Severe to life-threatening injuries (survival probable) |
| S3 | Life-threatening to fatal injuries (survival uncertain / fatal) |

# 2. Exposure (E)

| Class | Definition |
| --- | --- |
| E0 | Incredible |
| E1 | Very low probability (< 1% of operating time) |
| E2 | Low probability |
| E3 | Medium probability |
| E4 | High probability (default operating condition) |

# 3. Controllability (C)

| Class | Definition |
| --- | --- |
| C0 | Controllable in general |
| C1 | Simply controllable (≥ 99% of drivers/situations) |
| C2 | Normally controllable (90–99%) |
| C3 | Difficult to control or uncontrollable (< 90%) |

# 4. ASIL Determination (ISO 26262-3, Table 4 — condensed)

QM = Quality Managed (no ASIL). A is the lowest safety integrity level, D the highest.

| S | C1/E1 | C1/E2 | C1/E3 | C1/E4 | C2/E1 | C2/E2 | C2/E3 | C2/E4 | C3/E1 | C3/E2 | C3/E3 | C3/E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S1 | QM | QM | QM | QM | QM | QM | QM | A | QM | QM | A | B |
| S2 | QM | QM | QM | A | QM | QM | A | B | QM | A | B | C |
| S3 | QM | QM | A | B | QM | A | B | C | A | B | C | D |

# 5. Worked Classification — ACC vs. HA-LCA vs. VDD

| Feature | Severity | Exposure | Controllability | ASIL / QM | Notes |
| --- | --- | --- | --- | --- | --- |
| ACC – Longitudinal Speed/Gap Control | S2 (rear-end collision at moderate closing speed) | E4 (active on every ACC-enabled drive) | C2 (driver can brake/override within reaction time) | ASIL B | Comfort-only functions (REQ-ACC-001, 002) may be QM; gap-maintenance torque/brake requests (REQ-ACC-003, 004) carry the ASIL B allocation. |
| HA-LCA – Highway/Lane Display Mode Arbitration | S0 (no direct actuation — display/indicator output only) | E4 (evaluated continuously during Highway Assist/Lane Change operation) | C0 (no driver action required or affected by this logic in isolation) | QM | Only arbitrates which display mode value is written to the cluster/HMI; no direct actuation authority. If it later gates or informs a driver-facing lane-change decision, re-assess. |
| VDD – Vehicle Direction Determination | S0 (advisory indicator only — no actuation) | E4 (evaluated continuously while driving) | C0 (informational; driver retains full control regardless of indicator state) | QM | As specified, only produces a driver-facing direction indicator from steering yaw + camera confirmation. If later coupled with GPS routing, Lane Detection, or Wrong-Way Detection to gate a warning/intervention, re-assess at that integration point. |