# ISO 26262 / ASPICE Compliance Guidelines

Traceability expectations for MBD workflows · SWE.1–SWE.6 work product linkage

# 1. ISO 26262 Part 6 — Traceability Expectations

ISO 26262-6 (Product development at the software level) requires bidirectional traceability across the full software safety lifecycle. For an MBD workflow, this maps to:

Safety Requirement ↔ Software Requirement (REQ-xxx) — each software requirement traces to its parent safety goal / technical safety requirement (or to a quality requirement, for QM-classified elements such as HA-LCA).

Software Requirement ↔ Software Architecture — each requirement is allocated to one or more model subsystems.

Software Architecture ↔ Detailed Design / Model elements — subsystem-level blocks, Stateflow states, and SLDD entries trace to architectural elements.

Design ↔ Verification — every requirement has at least one MIL/SIL/PIL/HIL test case; ASIL C/D elements additionally require structural coverage (e.g., MC/DC).

Verification results ↔ Safety Case ↔ AUTOSAR mapping — closure evidence and AUTOSAR S-R interface mapping are archived and linked back to the originating requirement.

Note: In Simulink, this is implemented using Requirements Toolbox links (requirement → block → test case) and surfaced in the traceability/status report used for Model Advisor and safety/quality assessment reviews.

# 2. ASPICE SWE.1–SWE.6 — Work Product Linkage

| Process | Work Product Focus | MBD Artifact Linkage |
| --- | --- | --- |
| SWE.1 Software Requirements Analysis | Requirements analysis | Requirement document (REQ-xxx table), Simulink Requirements linkset, stakeholder review record |
| SWE.2 Software Architectural Design | Architecture design | Simulink model top-level architecture (subsystem partitioning), interface/SLDD definition, ASIL/QM allocation per subsystem |
| SWE.3 Software Detailed Design & Unit Construction | Detailed design | Subsystem-level Simulink model (Stateflow charts, block logic), Model Advisor 'MAAB compliance' report, naming/style guide conformance |
| SWE.4 Software Unit Verification | Unit verification | Model-in-the-Loop (MIL) test cases per requirement, Simulink Test / Requirements Toolbox traceability matrix, coverage report |
| SWE.5 Software Integration & Integration Test | Integration test | Software-in-the-Loop (SIL) / Processor-in-the-Loop (PIL) test results, generated C-code review, MISRA-C static analysis report |
| SWE.6 Software Qualification Test | Qualification test | Hardware-in-the-Loop (HIL) test results, requirement-based test closure report, final traceability sign-off (req → model → code → test) |

For the simulator, each program stage (see the Program Stage Taxonomy document) should be capable of reporting which SWE.x work products are complete, in-progress, or not started, so that ASPICE process-capability status can be derived automatically from stage/artifact completion.