# Program Stage Taxonomy

Simulator input field · 12 discrete program stages with entry activity and exit criterion

This is the suggested set of discrete stage values the agentic simulator can expose as a program-progress input/dropdown per project. Each stage has a clear entry activity and exit criterion so 'how far along the program is' can be captured as a single categorical field (optionally paired with a 0–100% completion slider within the active stage).

| # | Stage | Typical Activity | Exit Criterion |
| --- | --- | --- | --- |
| 1 | Requirements Definition | Draft, review, and baseline REQ-xxx set; SLDD signal list defined; stakeholder sign-off | Requirement doc reviewed & approved; traceability IDs assigned |
| 2 | Architecture & ASIL/QM Allocation | Subsystem partitioning, interface definition, S/E/C rated and ASIL (or QM) allocated per requirement/subsystem | Architecture diagram + allocation table approved |
| 3 | Model Design (MBD) | Simulink / Stateflow subsystem construction per architecture; naming & style guide applied | Model builds clean; no unresolved TODOs |
| 4 | SLDD & AUTOSAR Interface Mapping | SLDD created & linked; signals mapped to AUTOSAR Editor S-R interfaces (where AUTOSAR Coder is the target profile) | SLDD linked; all signals resolved in AUTOSAR Editor with no unmapped ports |
| 5 | Model Advisor / Static Checks | MBD compliance checks (MAAB/JMAAB), MISRA guideline checks on model, Model Advisor report | Zero Model Advisor errors; warnings dispositioned |
| 6 | Model-in-the-Loop (MIL) Test | Requirement-based test cases executed against the model in simulation | ≥ required requirement coverage; MC/DC for ASIL C/D elements |
| 7 | Code Generation | AUTOSAR Coder / Embedded Coder production code generation; code-to-model traceability report | Code generated with zero blocking errors; traceability report archived |
| 8 | Software-in-the-Loop / Processor-in-the-Loop (SIL/PIL) | Generated code executed in host / target-processor simulation against same test cases as MIL | Back-to-back MIL vs SIL/PIL numerical equivalence within tolerance |
| 9 | Static Code Analysis | MISRA-C:2012 compliance scan, code metrics, review of generated code deviations | No mandatory MISRA violations open; deviations documented & approved |
| 10 | Hardware-in-the-Loop (HIL) Test | Integration test on target ECU/HIL rig with plant + fault-injection scenarios | Requirement-based HIL test cases pass; safety mechanisms verified |
| 11 | Verification & Validation Closure | Full req → model → code → test traceability matrix closed; safety case / quality-record artifacts compiled | 100% requirement traceability closure; open items dispositioned |
| 12 | Release / Sign-off | Configuration baseline frozen, ASPICE work products archived, release note issued | Program stakeholder + safety sign-off recorded |

Note: Recommended simulator field — Stage (single-select, 1–12 above) + Stage Completion % (0–100). Optionally add a Blocked flag (Y/N) with free-text reason, since ASPICE/ISO reviews commonly stall a stage on open non-conformances rather than moving cleanly to the next one.