# Simulink Model Architecture

Repository Structure & Per-Feature Subsystem Breakdown (ACC · HA-LCA · VDD)

Project files shared across email as attachments (zip files).

# 1. Recommended Repository Layout (per project)

| Folder / File | Contents |
| --- | --- |
| /Requirements/ | REQ-xxx.xlsx or .docx, Simulink Requirements linkset (.slreqx) |
| /Model/<Feature>_TopModel.slx | Top-level architecture model – subsystem partitioning only |
| /Model/Subsystems/ | One .slx or model-reference per subsystem (see 2 – 4) |
| /Model/<Feature>.sldd | Simulink Data Dictionary – signals, parameters, enums |
| /AUTOSAR/ | AUTOSAR Editor S-R interface mapping (.arxml), component description |
| /Tests/MIL/ | Model-in-the-Loop test harness + .mldatx test cases |
| /Tests/SIL_PIL/ | SIL/PIL harness, back-to-back comparison scripts |
| /CodeGen/ | AUTOSAR Coder / Embedded Coder output, code generation report, traceability report |
| /StaticAnalysis/ | Model Advisor report (MAAB), MISRA-C static analysis report |
| /Docs/ | Architecture diagram, ASIL/QM allocation table, release notes |

# 2. ACC — Subsystem Breakdown

| Subsystem | Function | Key I/O |
| --- | --- | --- |
| Driver Input Handler | Set-speed, resume, cancel, time-gap selection | In: SetSpeed_Req_kph, TimeGap_Setting_s |
| Lead Vehicle Tracker | Validates and tracks lead-object range / rel-vel | In: LeadVehicle_Range_m, LeadVehicle_RelVel_mps |
| Longitudinal Controller (Speed Mode) | PID control to SetSpeed_Req_kph | Out: ThrottleTorque_Req_Nm |
| Longitudinal Controller (Gap Mode) | Time-gap following control | Out: Accel_Req_mps2 |
| ACC State Machine (Stateflow) | OFF/STANDBY/ACTIVE/OVERRIDE transitions | Out: ACC_State |
| Comfort Limiter | Clamps accel/decel to comfort bounds | Out: Accel_Req_mps2 (bounded) |

# 3. HA-LCA — Subsystem Breakdown

| Subsystem | Function | Key I/O |
| --- | --- | --- |
| Highway_DisplayUnit (Stateflow) | Condition/action logic mapping Highway_Input_Signal + Enable_Display + Input_Switch to Highway_Display_Mode_1–4 | In: Highway_Input_Signal, Enable_Display, Input_Switch → Out: Highway_Display_Mode_1–4 |
| Lane_DisplayUnit (Stateflow) | Condition/action logic mapping Lane_Input_Signal + Enable_Display + Input_Switch to Lane_Display_Mode_1–4 | In: Lane_Input_Signal, Enable_Display, Input_Switch → Out: Lane_Display_Mode_1–4 |
| Display Path Arbitration (Switch × 4) | Selects Highway vs. Lane display value per output, controlled by Highway_Input_Signal | In: Highway/Lane_Display_Mode_1–4 → Out: Display_Mode_1–4 |
| AUTOSAR RTE Interface Layer | Maps model ports to Rte_IRead_… / Rte_IWrite_… calls per AUTOSAR S-R interfaces | Generated by AUTOSAR Coder from AUTOSAR Editor mapping |

# 4. VDD — Subsystem Breakdown

| Subsystem | Function | Key I/O |
| --- | --- | --- |
| Steering Direction Classification (Switch blocks) | Compares SteeringWheel_YawDegreeInput against Right_Turn_AngularLimit / Left_Turn_AngularLimit / Straight_Drive_Steering_Angle | In: SteeringWheel_YawDegreeInput → Out: Vehicle_Turn_Status |
| Road Sign Confirmation (Compare + Logical Operator) | Compares Vehicle_Turn_Status against RightTurn_RoadSign / LeftTurn_RoadSign / Straight_RoadSign, cross-checks against CameraInput_RoadSign, resolves single output | In: Vehicle_Turn_Status, CameraInput_RoadSign → Out: Vehicle_Direction_Indicator |

Requirement 1 and Requirement 2 map 1:1 to these two subsystems, matching the tagging expectation stated in the mini-project brief. Code generation storage classes (if exercised): Input → ImportedExtern, Output → ExportToFile, Local → Localizable, Calibration → Const; sample time 0.01 s for all signals.