# Use Case 1 — Adaptive Cruise Control (ACC)

Sample Requirement Document · MBD Workflow (Requirement Tagging & Traceability, SLDD, Configuration, Model Advisor, Code Generation)

# Aim

To develop the Adaptive Cruise Control feature as per the requirement document using MATLAB Simulink. To follow all MBD-related processes such as Requirement Tagging & Traceability, SLDD creation, Configuration Parameter changes, Model Advisor check & Code Generation.

# Overview

ACC controls vehicle speed to a driver-set target when the lane ahead is clear, and automatically transitions to time-gap-based following control when a lead vehicle is detected, using a longitudinal controller bounded by comfort acceleration/deceleration limits.

# Requirement Tagging & Traceability Table

| Req ID | Requirement Description | Type | Traced Signal |
| --- | --- | --- | --- |
| REQ-ACC-001 | The system shall accept a driver Set-Speed input (SetSpeed_Req_kph) in the range 30–180 kph via steering-wheel stalk input. | System | SetSpeed_Req_kph |
| REQ-ACC-002 | WHEN no lead vehicle is detected, the system shall control vehicle speed to SetSpeed_Req_kph using the longitudinal PID controller. | System | ThrottleTorque_Req |
| REQ-ACC-003 | WHEN a lead vehicle is detected within Following_Range, the system shall maintain the driver-selected time-gap (1.0 / 1.4 / 1.8 / 2.2 s) by modulating throttle/brake request. | System | FollowingDistance_Req_m |
| REQ-ACC-004 | The system shall limit commanded longitudinal deceleration to 3.0 m/s² and commanded acceleration to 2.0 m/s² under normal ACC control (comfort limits). | Safety | Accel_Req_mps2 |
| REQ-ACC-005 | The system shall disengage ACC when the driver presses the brake pedal, presses Cancel, or when vehicle speed falls below 0 kph (full stop, non Stop & Go variant). | System | ACC_State |
| REQ-ACC-006 | The system shall re-engage from Stand-by to Active state on Resume switch press, provided lead-vehicle object validity is TRUE. | System | ACC_State |
| REQ-ACC-007 | The system shall tag Requirement REQ-ACC-003 and REQ-ACC-004 to the Longitudinal Controller subsystem for Model Advisor traceability review. | Traceability | N/A |
| REQ-ACC-008 | The system shall run Model Advisor 'MBD compliance' checks (naming, unconnected ports, data type consistency) with zero warnings prior to code generation. | Process | N/A |

# Simulink Data Dictionary (SLDD) — Key Signals

| Signal Name | Data Type | Range / Units | Source |
| --- | --- | --- | --- |
| SetSpeed_Req_kph | uint8 | 30 – 180 kph | Driver Stalk Input |
| LeadVehicle_Range_m | single | 0 – 150 m | Radar/Camera Fusion ECU |
| LeadVehicle_RelVel_mps | single | -40 – 40 m/s | Radar/Camera Fusion ECU |
| TimeGap_Setting_s | enum | {1.0, 1.4, 1.8, 2.2} | Driver HMI Switch |
| ThrottleTorque_Req_Nm | single | 0 – 350 Nm | Internal – output to Powertrain ECU |
| ACC_State | enum | OFF/STANDBY/ACTIVE/OVERRIDE | Internal – state machine |